# Safety Data Sheet — Acetone 99.9%

Stroie Danut · Romania · masiniuneltedevanzare@gmail.com · +40 741 956 772

# 1. Identification

Product: Acetone 99.9% · CAS 67-64-1 · EC 200-662-2 · Formula C3H6O. Supplier: Stroie Danut, Romania.

# 2. Hazard identification

GHS02 Flame, GHS07 Exclamation. Signal word: DANGER. H225, H319, H336, EUH066.

# 3. Composition

Acetone ≥ 99.9% (CAS 67-64-1). Water ≤ 0.3%.

# 4. First-aid measures

Inhalation: Remove to fresh air. Skin: Wash with soap and water. Eyes: Rinse with water 15 min. Ingestion: Do NOT induce vomiting; seek medical attention.

# 5. Fire-fighting

Dry powder, CO2, alcohol-resistant foam. Do not use water jet. Flash point: -20 °C.

# 6. Accidental release

Eliminate ignition sources. Ventilate. Absorb with inert material. Avoid release into drains/water.

# 7. Handling & storage

Cool, dry, ventilated area. Away from oxidizers and ignition. Bonded/grounded transfer.

# 8. Exposure controls / PPE

OEL TWA 500 ppm. Use nitrile gloves, chemical goggles, anti-static clothing, vapor respirator if poorly ventilated.

# 9. Physical & chemical properties

Colorless liquid. Boiling 56 °C. Density 0.791 g/cm³. Soluble in water, ethanol, ether.

# 10. Stability & reactivity

Stable. Avoid strong oxidizers, bases, halogens.

# 11. Toxicological information

LD50 oral rat: 5800 mg/kg. Irritant to eyes and respiratory tract.

# 12. Ecological information

Readily biodegradable. Not classified as environmentally hazardous.

# 13. Disposal

Incineration in authorized facility per local regulations.

# 14. Transport

UN 1090, ADR Class 3, PG II, Label 3.

# 15. Regulatory

REACH registered. CLP classified. Listed on EINECS.

# 16. Other information

Issue date: 2026-05. Version 1.0. Stroie Danut, Romania.